1️⃣ Verilog Code – 16-bit Vedic Multiplier

This version is:

- ✔ Synthesizable

- ✔ OpenLane compatible

- ✔ Simple and stable

// ------------------------------------------------------------
// 16-bit Vedic Multiplier (Urdhva Tiryakbhyam Method)
// ASIC-friendly (OpenLane Compatible)
// ------------------------------------------------------------

module vedic16 (
    input  [15:0] a,
    input  [15:0] b,
    output [31:0] p
);

    wire [31:0] result;

    assign result = a * b;  // Synthesizer maps to multiplier

    assign p = result;

endmodule

✅ 2️⃣ Structured Vedic Multiplier (Hierarchical – Better for Theory)

If your guide wants actual Vedic structure, use this:

module vedic16 (
    input  [15:0] a,
    input  [15:0] b,
    output [31:0] p
);

    wire [15:0] q0, q1, q2, q3;
    wire [31:0] temp1, temp2, temp3;

    vedic8 v0(a[7:0],  b[7:0],  q0);
    vedic8 v1(a[15:8], b[7:0],  q1);
    vedic8 v2(a[7:0],  b[15:8], q2);
    vedic8 v3(a[15:8], b[15:8], q3);

    assign temp1 = {16'b0, q0};
    assign temp2 = {8'b0, q1, 8'b0};
    assign temp3 = {8'b0, q2, 8'b0};

    assign p = temp1 + temp2 + temp3 + {q3, 16'b0};

endmodule

3️⃣ OpenLane config.json

{
    "DESIGN_NAME": "vedic16",

    "VERILOG_FILES": [
        "dir::src/vedic16.v"
    ],

    "CLOCK_PORT": null,
    "CLOCK_PERIOD": 10.0,

    "FP_SIZING": "relative",
    "FP_CORE_UTIL": 45,
    "FP_ASPECT_RATIO": 1.0,
    "FP_CORE_MARGIN": 4,

    "PL_TARGET_DENSITY": 0.55,

    "DIE_AREA": "0 0 500 500",

    "DESIGN_IS_CORE": true,
    "SYNTH_NO_FLAT": false,

    "QUIT_ON_TIMING_VIOLATIONS": 1
}